Part A — Model

Goal: automatically find existing bike lane segments that are on bike-friendly terrain using the slope raster.

Do not use the road layer in this model. Roads are for a later optional Part B.

## Before starting: create a new clean model

Go to:

Processing → Graphical Modeler

Create a new model:

Model name:

Bike Friendly Terrain Analysis Clean

Group:

Portfolio Models

Save it.

# Step 1 — Add model input: Slope Raster

In the left panel, go to Inputs.

Add:

Raster layer

Name it:

Slope Raster

This is your existing slope.tif.

Why we do this:
The slope raster stores terrain steepness. Each pixel has a slope value in degrees. We use it to decide what terrain is comfortable for biking.

Save the model.

# Step 2 — Add model input: Bike Lanes

Add another input:

Vector layer

Name it:

Bike Lanes

Geometry type can be:

Line

This is your bike_utm.gpkg.

Why we do this:
Bike lanes are the transportation features we want to evaluate. The final output will be a filtered version of these bike lanes.

Save the model.

# Step 3 — Buffer Bike Lanes

Search in algorithms:

Buffer

Add:

Vector geometry → Buffer

Set:

Input layer: Bike Lanes
Distance: 50
Segments: 5
Dissolve result: checked
Output: bike_lane_buffer_50m

Why we do this:
Instead of analyzing the full huge DEM/slope raster, we create a 50-meter area around the bike lanes. This makes the model much faster because QGIS only processes terrain near bike lanes.

Save the model.

# Step 4 — Clip Slope Raster to Bike Lane Buffer

Search:

Clip raster by mask layer

Add:

GDAL → Clip raster by mask layer

Set:

Input raster: Slope Raster
Mask layer: bike_lane_buffer_50m
Target CRS: EPSG:26918
Output: slope_clipped_to_bike_area

Why we do this:
This cuts the slope raster down to only the area around bike lanes. It reduces file size, processing time, and the number of polygons created later.

Save the model.

# Step 5 — Raster Calculator: Find Bike-Friendly Slope

Search:

Raster calculator

Use the GDAL raster calculator if possible.

Set the input raster to:

slope_clipped_to_bike_area

Number of raster band for A:

1

Expression:

A <= 15

Output:

slope_bike_friendly_15

Output type:

Byte

Why we do this:
This creates a binary raster. Pixels with slope less than or equal to 15 degrees become 1, meaning bike-friendly. Pixels above 15 degrees become 0, meaning not bike-friendly.

Save the model.

# Step 6 — Polygonize the Bike-Friendly Raster

Search:

Polygonize

Add:

GDAL → Polygonize raster to vector

Set:

Input layer: slope_bike_friendly_15
Band: 1
Field name: DN
Output: slope_bike_friendly_15_polygon

Why we do this:
Raster data is made of pixels. Bike lanes are vector lines. To compare them spatially, we convert the bike-friendly raster pixels into vector polygons.

Save the model.

# Step 7 — Keep Only DN = 1

Search:

Extract by attribute

Add:

Vector selection → Extract by attribute

Set:

Input layer: slope_bike_friendly_15_polygon
Attribute: DN
Operator: =
Value: 1
Output: slope_bike_friendly_15_only

Why we do this:
Polygonize creates polygons for both 1 and 0. We only want the 1 polygons, because those represent bike-friendly terrain.

Save the model.

# Step 8 — Extract Bike Lanes on Bike-Friendly Terrain

Search:

Extract by location

Add:

Vector selection → Extract by location

Set:

Input layer / Extract features from: Bike Lanes
Predicate: intersect
Intersect layer / Compare to: slope_bike_friendly_15_only
Output: bike_friendly_segments_15

Why we do this:
This finds bike lane features that touch or overlap the bike-friendly terrain polygons. The result is your final decision-ready layer.

Save the model.